DCCN Semester Project

(Banking System Network Design & Implementation)

As a part of your 3rd-semester networking project, you are required to design and implement the “Banking Network System”. Adamjee Company Ltd. is a PK-owned company that deals with Banking & Insurance. The company is intending to expand its services across the continent having the first branch to be in Islamabad, Pakistan. The company has secured a four-story building to operate within the Islamabad capital city. Therefore, the company would like to allow sourcing the knowledge from a group of second-year students from the local university to design and implement their company network. Carefully read down the requirements then model the design and implement the network based on the company’s needs. Each floor has departments as provided in the table below.

| First Floor |  |  |  |
| --- | --- | --- | --- |
| No. | Departments | No. of PC | No. of Printers |
| 1 | Management | 20 | 4 |
| 2 | Research | 20 | 4 |
| 3 | HR | 20 | 4 |

| Second Floor |  |  |  |
| --- | --- | --- | --- |
| No. | Departments | No. of PC | No. of Printers |
| 1 | Marketing | 20 | 4 |
| 2 | Accounting | 20 | 4 |
| 3 | Finance | 20 | 4 |

| Third Floor |  |  |  |
| --- | --- | --- | --- |
| No. | Departments | No. of PC | No. of Printers |
| 1 | Logistics & Store | 20 | 4 |
| 2 | Customer care | 20 | 4 |
| 3 | Guest Area | 40 | 2 |

| Fourth Floor |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Departments | No. of PC | No. of Printers | No. of Servers |
| 1 | Administration | 20 | 2 | N/A |
| 2 | ICT | 20 | 4 | N/A |
| 3 | Server Room | 2 | 4 | 3 (DHCP, HTTP & Email) |

Requirements:

1.	Use a software modeling tool to visualize the network topology.

- Software Modelling Tools: MS Visio, Visual paradigm, or Draw.io for modeling network design.

2.	Use any of the following network simulation software to implement the above topology:

- Simulation software: Cisco Packet tracer or GNS3 for design & implementation.

- There should be one router on each floor. The router should be connecting switches on that floor.

- Use OSPF as the routing protocol to advertise routes.

- Each department is required to have a wireless network for the users.

- Each department except the server room will be anticipated to have around 60 users both wired and wireless users.

- Host devices in the network are required to obtain IPV4 addresses automatically.

- Devices in all departments are required to communicate with each other.

- All devices in the network are expected to obtain an IP address dynamically from the dedicated DHCP servers located in the server room.

- Create HTTP and E-mail servers.

- Configure SSH in all the routers for remote login.

3. Configure the basic configuration of the devices:

- Hostnames

- Line Console & VTY passwords

- Disable domain IP lookup

4. Each department should be in a different VLAN

- Create VLANs in every department

- VLAN 10,20,30……. Etc.

- Each VLAN should be a different subnetwork.

5. Planning of IP addresses:

- You have been given 192.168.10.0 as the base address for this network.

- Do subnetting based on the number of hosts in every department.

- Identify the subnet mask, useable IP address range, and broadcast address for each subnet.

6. End Devices Configurations:

- Configure all the end devices in the network with the appropriate IP address based on the calculations above.

7. Configure port security:

- Use the sticky command to obtain the MAC address.

- Violation mode of the shutdown.

8. Test Communication:

- Do devices in the same VLAN communicate?

- Do the devices in different VLANs communicate?